UNIT IV                   ELECTRO MAGNETIC CIRCUIT

## OBJECTIVES

When electric current passes through current carrying conductor or coil then a magnetic field is produced around it. The working of appliances like electric bell is based on this principle. As opposite to this if a  continuous change in magnetic  field is produced then electric current can be produced. This is how electricity and magnetism have become synonymous today. After studying this lesson you will be able to:

- Explain the concept of magnetic field and state the properties of lines of magnetic force;

- Infer that when electricity flows through a conductor, magnetic field is produced around it ;

- Explain the force experienced by a current carrying conductor placed in a magnetic field;

  - . describe electromagnetic induction and its importance in different aspects of daily life;

.

4.1

CONCEPT OF ELECTROMAGNETIC FIELD PRODUCED BY FLOW OF ELECTRIC CURRENT

4.1.1Concept of Electric Charge

Electric Charge is the fundamental property of matter that develop electrical influence. Electric charge can be positive or negative, stationary or moving one.  Electric charge is the that property of matter due to which it exhibit electric, magnetic and electromagnetic effects.

Properties of electric charge:

- Total electric charge in an isolated system remains constant.

- Electric charge is a scalar quantity and is always associated with mass.

- Electric charge at rest causes an electric field, while moving at a constant speed it causes a magnetic field and when accelerated it emits electromagnetic waves.

- Like charges repel each other and unlike charges attract each other.

- SI unit of charge is coulomb (C ). One coulomb is the quantity of charge that has passed through a conductor carrying ine ampere current in one second.

Coulomb’s Law:

Coulom’s law states that “the magnitude of the electric force between two charges is directly proportional to the product of the two charges and inversely proportional to the square of the distance between them.”

Let two charges q1 and q2 present in vacuum at points A and B at a distance “d” as shown in fig 4.1.

Q1                            Q2                                              Q1                                Q2

+

+

-

+

D							D

Fig: 4.1	a) Repulsion							b) Attraction

The force between two charges is given by:

F α q1.q2/d2				---(1)

F= K q1q2/d2				--(2)

Where K is constant whose magnitude depends on the medium in which charges are kept.

K=1/4πԐoԐr

Where Ԑo=absolute permittivity of vacuum=8.854×10-12  F/m

Ԑr= relative  permittivity of medium for vacuum or airand is equal to 1.

F=  q1.q2/4πԐoԐr d2				---(3)

F=  q1.q2/4π 8.854×10-12Ԑr d2

F= 9×109q1.q2/Ԑr d2Newtons ( in medium)

F= 9×109q1.q2/ d2Newtons ( in air)		---(4)

When q1andq2 are equal then we can determine the unit of charge.

Q1  			Q2

D

Q1		Q2

d

Fig 4.2	Charge in medium			Fig 4.3       Charge in air

If in equation (4) 	Q1=Q2=Q, and d=1m

F=9×109Newtons

Then Q2=1

Or Q=+-√1 coulombs

Hence 1 coulomb is that charge when placed at a distance of 1from equal and similar charge, repels it with a force of 9×109Newtons.

The branch of science associated with the study of effects of stationary charges is called electrostatics.

Example 1 From PEE page no 3-3

Example 2 From PEE page no 3-3

4.1.2 Concept of Magnetism

A force of attraction  or repulsion that acts  at a distance and caused due to a magnetic field is called magnetism.

The branch of science concerned with the forces that occur between electricity charged particles is called electromagnetism.

Two aspects of electromagnetism are:

i)  Electrcity

ii) Magnetism

The magnetic field is a force created by moving electric charges and magnetic dipoles. It is a vector field. The magnetic force is force of attraction or repulsion between electrically charged particles because of their motion.

Electricity and magnetism are closely related. spinning magnets cause an electric current to flow and flowing electron produces a magnetic field. The relationship between electricity and magnetism is called electromagnetism. Magnets are artificial magnets and natural magnets. Artificial magnets were formerly made by rubbing iron with a natural magnet. The iron is behaving as a natural magnet is known as magnetite (Fe3O4). A piece of magnetite is called a natural magnet.

Permanent magnets have the property of retaining magnetism indefinitely and they require no electric aid to magnetize like existing coils(ampere turns).

Ampere Turns

Ampere turns is the product of current I flowing through a coil in amperes and the number of turns of coil.

It is represented by AT. It is the magneto motive force (m.m.f.) produced by the coil.

Artificial magnets are made by process of magnetization. A soft iron piece if similarly magnetized will remain its magnetism only for a small time.

Magnetic Material

Substances which are attracted by a magnet are called magnetic materials. Iron, steel and some of their alloys are superior to all other metals. Cobalt, nickel and some of their alloys possess certain magnetic properties.

## 4.1.3 MAGNETIC FIELD

Keep a small magnetic needle near a bar magnet. The magnetic needle rotates and stops in a particular direction only. This shows that a force acts on the magnetic needle that makes it rotate and rest in a particular direction only. This force is called torque.

The region around the magnet where the force on the magnetic needle occurs and the needle stops at a specific direction, is called a magnetic field. The direction of magnetic field is represented by magnetic line of forces. As shown in Fig. 4.4,the direction of magnetic needle changes continuously and it takes the curved path while moving from north to south. This curved path is known as magnetic line of forces. Tangent line draw at any point on magnetic line of force, represent the direction of magnetic field at that point.

.If the lines along which the iron fillings set themselves near the ends of the bar magnet are extended into the magnet, they would be seen to concentrate themselves in two particular regions near each end of the magnet. These regions are called as poles of the magnet.

The imaginary line joining the two poles of a magnet is known as magnetic –axis and a plane taken at right angles to the magnetic axis and through a point midway between the poles, is called the neutral zone or equator of the magnet.

These magnetic line of forces have following properties:

1.Magnetic line of forces always starts from North Pole and end at South Pole of  the magnet.

2.These lines of forces never intersect each other.

3.Near the poles magnetic lines are very near to each other which shows that magnetic field at the poles is stronger as compare to other parts.

4. They seek the path of least resistance between opposite magnetic poles. In a single bar magnet as shown to the right, they attempt to form closed loops from pole to pole.

5. They all have the same strength.

6.Their density decreases (they spread out) when they move from an area of higher permeability to an area of lower permeability

Fig. 4.4

Our Earth itslef acts as a giant magnet with south magnetic pole somewhere in the Arctic and north magnetic pole in Antaractic. The Earth also behaves like a bar magnet. It’s hot liquid centre core contains iron and as it moves, it creates an electric current that cause a magnetic field around the Earth. The Earth has a north and south magnetic pole. These poles are not same with the geographic north and south poles on a map and tilted at an angle of 11.3 degree with respect to it. Due to this, if a magnetic needle is suspended freely, it rests in the north-south direction and is useful for navigation. 	Fig 4.5.

# 4.1.4 1Magnetic Flux

Magnetic flux is a measurement of the total magnetic field which passes through a given area. It is a useful tool for helping describe the effects of the magnetic force on something occupying a given area. The measurement of magnetic flux is tied to the particular area chosen. We can choose to make the area any size we want and orient it in any way relative to the magnetic field.

If we use the field-line picture of a magnetic field then every field line passing through the given area contributes some magnetic flux. The angle at which the field line intersects the area is also important. A field line passing through at a glancing angle will only contribute a small component of the field to the magnetic flux. When calculating the magnetic flux we include only the component of the magnetic field vector which is normal to our test area.

If we choose a simple flat surface with area AAA as our test area and there is an angle \thetaθtheta between the normal to the surface and a magnetic field vector (magnitude BBB) then the magnetic flux is,

ΦB=BAcosθ

Phi, equals, B, A, cosine, theta

In the case that the surface is perpendicular to the field then the angle is zero and the magnetic flux is simply B ABAB, A. Figure 1 shows an example of a flat test area at two different angles to a magnetic field and the resulting magnetic flux.

Fig 4.6

## 4.1.4 The Magnitude of Magnetism

We now know that the lines of force or more commonly the magnetic flux around a magnetic material is given the Greek symbol, Phi, ( Φ ) with the unit of flux being the Weber, ( Wb ) after Wilhelm Eduard Weber. But the number of lines of force within a given unit area is called the “Flux Density” and since flux ( Φ ) is measured in ( Wb ) and area ( A ) in metres squared, ( m2 ), flux density is therefore measured in Webers/Metre2 or ( Wb/m2 ) and is given the symbol B.

However, when referring to flux density in magnetism, flux density is given the unit of the Tesla after Nikola Tesla so therefore one Wb/m2 is equal to one Tesla, 1Wb/m2 = 1T. Flux density is proportional to the lines of force and inversely proportional to area so we can define Flux Density as:

### 4.1.5 Magnetic Flux Density

The symbol for magnetic flux density is B and the unit of magnetic flux density is the Tesla, T.

It is important to remember that all calculations for flux density are done in the same units, e.g., flux in webers, area in m2 and flux density in Teslas.

4.1.6 ELECTROMAGNETISM

Electromagnetism is the science that deals with the phenomena associated with electric and magnetic fields and their interaction with each other, with electric charges and currents.

An electromagnet is a type of magnet in which magnetic field is produced by the flow of elctric current. For making electromagnet take a piece of paper and give a cylindrical shape.

Make several turns of a copper wire over this from one end to the other. This is called solenoid a long thin loop of wire. When the ends of the copper wires are attached to the ends of a battery (+ and –) current starts flowing through the coil which starts functioning as a bar magnet.

When the flow of current is stopped from the battery, then, its magnetic property ceases. If the +ve and –ve terminals of the battery are reversed, then the poles of the magnet are also reversed.

Fig.4.8Solenoid

For increasing the magnetic field, put soft iron like iron nails inside the core. So Current carry solenoid with soft iron core inside it forms an electromagnet. Electromagnet may be made as strong as one may desire.

Electromagnet are widely used as a components in electrical devices such as motor, generator, electrical bells MRI machine etc. Beside that strong electromagnet are also being used in break system of the superfast train in the world, in the cyclotrons and in mega experiments like experiment at CERN laboratory at Geneva. The comparison of a bar magnet and an electromagnet has been illustrated in the table given below.

4.1.7 MAGNETIC FIELD AROUND THE CURRENT CARRYING WIRE

If an electric current is made to flow in a wire, magnetic field produce arround it. For seeing this takes a conducting wire (like copper). Now with the help of connecting wires attach this to the two ends of a battery. Keep a magnetic needle parallel to the conducting copper wire as shown in Fig. 4.9(a). When the circuit is complete the magnetic needle shows deflection. This shows that when electric current flows through a conductor, magnetic field is produced around the conductor. If the current is increased, there is greater amount of deflection. If the direction of flow of electric current is changed (by reversing the end of the battery) the direction of deflection in the magnetic needle is also reversed. If the current flow is stopped the deflection in the magnetic needle also ceases. Thus magnetic field is an effect of flow of electric current through conducting wire. In the year 1820 a scientist from Denmark named

H.C. Oersted observed this effect for the first time.

Fig.4.9			(a)				(b)

The principle of the magnetic effect  of electric current used in many appliances like motor etc.

### 4.1.8  Magnetic field Due to a Current Carrying Conductor

A current carrying straight conductor has magnetic field in the form of concentric circles around it. Magnetic field of current carrying straight conductor can be shown by magnetic field lines.

The magnetic field of an infinitely long straight wire can be obtained by applying Ampere's law. The expression for the magnetic field is

Where

I is the current

r is the radial distance

The permeability of free space is

The direction of magnetic field through a current carrying conductor depends upon the direction of flow of electric current. The direction of magnetic field gets reversed in case of a change in the direction of electric current.

Let a current carrying conductor be suspended vertically and the electric current is flowing from south to north. In this case, the direction of magnetic field will be anticlockwise. If the current is flowing from north to south, the direction of magnetic field will be clockwise.

##### Right Hand Thumb Rule

Fig 4.10  (a)							(b)

Fig 4.11

The direction of magnetic field, in relation to direction of electric current through a straight conductor can be depicted by using the Right Hand Thumb Rule. It is also known as Maxwell’s Corkscrew Rule.

If a current carrying conductor is held by right hand, keeping the thumb straight and if the direction of electric current is in the direction of thumb, then the direction of wrapping of other fingers will show the direction of magnetic field.

As per Maxwell's corkscrew rule, if the direction of forward movement of screw shows the direction of current, then the direction of rotation of screw shows the direction of magnetic field.

Properties of Magnetic Field:

- The magnitude of magnetic field increases with increase in electric current and decreases with decrease in electric current.

- The magnitude of magnetic field, produced by electric current, decreases with increase in distance and vice-versa. The size of concentric circles of magnetic field lines increases with distance from the conductor, which shows that magnetic field decreases with distance.

- Magnetic field lines are always parallel to each other.

- No two field lines cross each other.

4.1.9 Magnetic field due to currents in two parallel conductors

From PEE book page no 3-17  and 3-18

Example 6 and 7 From PEE book page no 3-18

### 4.1.10 Magnetic Field Lines in a Current-Carrying Circular Loop

We know that the magnetic field lines have a circular pattern when an electric current is passed through a straight wire. However, when a wire is bent into a circular loop and an electric current is passed through it, the nature of the magnetic field lines changes as shown in the below illustration.

Fi: 4.14

Suppose, we take a circular coil with multiple turns, place it halfway through a sheet of cardboard, and connect it in series with the ammeter, battery, plug key, and a rheostat. Now sprinkle some iron fillings over the cardboard and pass an electric current through the wire by switching on the plug key. The iron fillings present on the cardboard will align in a specific pattern, proving the presence of a magnetic field.

Fig:4.15

- The magnetic field lines are concentric circles at every point of a current-carrying circular loop

- The magnetic field lines are straight at the center of the loop and the lines of force are in the same direction

- The magnetic field lines at the center of the loop are perpendicular to the plane of the loop carrying current

- At the center of the loop, the intensity of the magnetic field is maximum while the magnetic field intensity decreases as we move away from the center of the loop

- Each segment of the circular loop carrying current produces magnetic field lines in the same direction within the loop

- The direction of the magnetic field of every section of the circular loop can be found by using the right-hand thumb rule. As per the rule, If you are holding a current-carrying straight conductor in your right hand such that the thumb points towards the direction of current. Then your fingers will wrap around the conductor in the direction of the field lines of the magnetic field. That means when the electric current is in the upward direction around the axis of the circular conductor, the magnetic field lines are in an anti-clockwise direction while with the downward flow of electric current, the magnetic field lines are in a clockwise direction.

The magnitude of magnetic field is directly proportional to magnitude of current flowing through the coil and is inversely proportional to the radius of coil.

B α I

B α1/r

4.1.11 Magnetic Field due to Solenoid

Fig:4.16

A solenoid is a coil of wire designed to create a strong magnetic field inside the coil. By wrapping the same wire many times around a cylinder, the magnetic field due to the wires can become quite strong. If a current is passed through a solenoid, it produces a magnetic field. The lines of force run approximately parallel to the axis of the solenoid until they reach the ends where they begin to diverge. The number of turns N refers to the number of loops the solenoid has. More loops will bring about a stronger magnetic field. The formula for the field inside the solenoid is

B = I N / L

Where, B is the magnetic field

I is the current in the coil

N is the number of turns in the solenoid

L is the length of the coil

This formula can be accepted on faith; or it can be derived using Ampere's law as follows. Look at a cross section of the solenoid.

The magnetic field outside a solenoid is:

B=μ0 nl

Since the field outside the solenoid is comparatively less than that it is present inside where, we can consider it as zero as the length of the solenoid increases, and hence B = 0.

Total flux of solenoid:

In a long solenoid the flux density is uniform over the entire cross sectional area A so that the total flux

ϕ= B.A =(I N / L).A                              ---(in air)

ϕ= B.A =(I N / L).A                              ---(in medium)

Example 3 From PEE book page 3-14

Example 4 From PEE book page 3-14

4.1.12 Topic Force on a current carrying conductor in a magnetic field:

From PEE book page 3-14

4.1.13 Another definition of unit current and example from PEE book page 3-15

## 4..1.14 Magnetic Circuit Analysis:

## Definition of Magnetic Ciruit

From PEE book page 3-21 and 3-22 with diagram

## The presence of charges in space or in a medium creates an electric field, similarly the flow of current in a conductor sets up a magnetic field. Electric field is represented by electric flux lines, magnetic flux lines are used to describe the magnetic field. The path of the magnetic flux lines is called the magnetic circuit. Just as a flow of current in the electric circuit requires the presence of an electromotive force, so the production of magnetic flux requires the presence of magneto-motive force (mmf). We now discuss some properties related to magnetic flux.

### (a) Flux density (B):

The magnetic flux lines start and end in such a way that they form closed loops. Weber (Wb) is the unit of magnetic flux (Φ). Flux density (B) is the flux per unit area. Tesla (T) or Wb/m2 is the unit of flux density.

where B is a quantity called magnetic flux density in teslas, Φ is the total flux in webers and A is the area perpendicular to the lines in m2.

### (b) Magneto-motive force MMF (ζ):

A measure of the ability of a coil to produce a flux is called the magneto-motive force. It may be considered as a magnetic pressure, just as emf is considered as an electric pressure. A coil with N turns which is carrying a current of I amperes constitutes a magnetic circuit and produces an mmf of NI ampere turns. The source of flux (Φ) in the magnetic circuit is the mmf. The flux produced in the circuit depends on mmf and the length of the circuit.

### (c) Magnetic field strength / Field Strength(H):

The magnetic field strength of a circuit is given by the mmf per unit length.

The magnetic flux density (B) and its intensity (field strength) in a medium can be related by the following equation

where

- μ = μ0μr is the permeability of the medium in Henrys/metre (H/m),

- μ0 = absolute permeability of free space and is equal to 4π x 10-7 H/m and

- μr = relative permeability of the medium.

Relative permeability is a non-dimensional numeric which indicates the degree to which the medium is a better conductor of magnetic flux as compared to free space. The value of μr = 1 for air and non-magnetic materials. It varies from 1,000 to 10,000 for some types of ferro-magnetic materials.

### (d) Reluctance (R):

It is the property of the medium which opposes the passage of magnetic flux. The magnetic reluctance is analogous to resistance in the electric circuit. Its unit is AT/Wb. Air has a much higher reluctance than does iron or steel. For this reason, Magnetic Circuit Analysis used in electrical machines are designed with very small air gaps.

According to definition, reluctance = mmf/flux

The reciprocal of reluctance is known as permeance 1/R = Φ/ζ

Thus reluctance is a measure of the opposition offered by a magnetic circuit to the, setting up of the flux. The reluctance of the magnetic circuit is given by R = 1/μ 1/a.

where I is the length, a is the cross-sectional area of the Magnetic Circuit Analysis and it is the permeability of the medium.

From the above equations

Permeance(P)

It is reciprocal of reluctance and it is the readiness with which magnetic flux is developed. It is analogous to conductance in an electric circuit. Its unit is weber per ampere turn.

Thus P=Φ/ζ    ( Flux  /m.m.f.  )

4.1.15 Analogy between Magnetic and Electric Circuits:

### Similarities:

Similarities between electrical and magnetic circuits are given below.

| Magnetic circuit | Electric circuit |
| --- | --- |
| The close path for the magnetic flux is called magnetic circuit. | The close path for the electric current is called electric circuit. |
| The number of magnetic lines of force decide the magnetic flux. | Flow of electron decide the current passing through the conductor. |
| Flux (ф) = MMF/Reluctance | Current (I) = EMF/Resistance |
| MMF is the driving force in the magnetic circuit. The unit of MMF is ampere-turns (AT). | EMF is the driving force in the electric circuit. The unit of EMF is volts (V). |
| Reluctance (S) opposed by the magnetic path to the flux. The unit of reluctance is AT/wb. | Resistance (R) oppose the flow of current. The unit of resistance is ohm (Ω). |
| Flux (ф) measured in weber (Wb). | Current (I) measured in Ampere (A). |
| MMF measured in Amp Turns (AT). | EMF measured in Volts (V). |
| Permanence = 1/Reluctance = 1/S | Conductance = 1/Resistance = 1/R |
| Permeability (μ) | Conductivity (σ) |
| Reluctivity | Resistivity |
| Flux Density (B) = ф/A (Wb/m2) | Current Density (J) = I / A (A/m2) |
| Magnetic Intensity (H) = NI/L (AT/m) | Electric Intensity (E) = V/d (V/m) |
| Kirchhoff MMF Law and Flux Law is applicable to the Magnetic Flux. | Kirchhoff Current Law (KCL) and Kirchhoff Voltage Law (KVL) is applicable to the electrical circuit. |

### Dissimilarities:

There are few dissimilarities between the two circuits which are listed below:

| Magnetic circuit | Electric circuit |
| --- | --- |
| There is no magnetic insulator as flux can pass through all the materials, even through the air as well. For magnetic flux there is no perfect insulators. | There are many materials which used as insulators. From which current cannot pass. i.e. air, PVC, glass, synthetic resin etc. |
| Magnetic flux does not flow but it set up in the magnetic circuit. | The electric current actually flows in an electric circuit. |
| At constant temperature, the reluctance (S) of a magnetic circuit is not constant but varies with μr. | At constant temperature, the resistance of an electric circuit is constant as its value depends on resistivity which is almost constant. |
| Once magnetic flux is setup in a magnetic circuit, no energy needed. | Energy needed as long as current flows through electric circuit. |

# 4.2.1 Faraday’s Laws of Electromagnetic Induction

Faraday’s law of electromagnetic induction (referred to as Faraday’s law) is a basic law of electromagnetism predicting how a magnetic field will interact with an electric circuit to produce an electromotive force (EMF). This phenomenon is known as electromagnetic induction.

Faraday’s law states that a current will be induced in a conductor which is exposed to a changing magnetic field. Lenz’s law of electromagnetic induction states that the direction of this induced current will be such that the magnetic field created by the induced current opposes the initial changing magnetic field which produced it. The direction of this current flow can be determined using Fleming’s right-hand rule.

Faraday’s law of induction explains the working principle of transformers, motors, generators, and inductors. The law is named after Michael Faraday, who performed an experiment with a magnet and a coil. During Faraday’s experiment, he discovered how EMF is induced in a coil when the flux passing through the coil changes.

## Faraday’s First Law

Any change in the magnetic field of a coil of wire will cause an emf to be induced in the coil. This emf induced is called induced emf and if the conductor circuit is closed, the current will also circulate through the circuit and this current is called induced current.
Method to change the magnetic field:

- By moving a magnet towards or away from the coil

- By moving the coil into or out of the magnetic field

- By changing the area of a coil placed in the magnetic field

- By rotating the coil relative to the magnet

## Faraday’s Second Law

It states that the magnitude of emf induced in the coil is equal to the rate of change of flux that linkages with the coil. The flux linkage of the coil is the product of the number of turns in the coil and flux associated with the coil.

## Faraday Law Formula

Consider, a magnet is approaching towards a coil. Here we consider two instants at time T1 and time T2.

Flux linkage with the coil at time,T1 = NΦ1

Flux linkage with the coil at time,T1 = NΦ2

Change in flux linkage,N(Φ2 – Φ1)

Let this change in flux linkage be,Φ = Φ2 – Φ1

So, the Change in flux linkage NΦ

Now the rate of change of flux linkageNΦ/t

Take derivative on right-hand side we will getN dΦ/dt

The rate of change of flux linkage

But according to Faraday’s law of electromagnetic induction, the rate of change of flux linkage is equal to induced emf.

Considering Lenz’s Law.

Where:

- Flux Φ in Wb = B.A

- B = magnetic field strength

- A = area of the coil

## How To Increase EMF Induced in a Coil

- By increasing the number of turns in the coil i.e N, from the formulae derived above it is easily seen that if the number of turns in a coil is increased, the induced emf also gets increased.

- By increasing magnetic field strength i.e B surrounding the coil- Mathematically, if magnetic field increases, flux increases and if flux increases emf induced will also get increased. Theoretically, if the coil is passed through a stronger magnetic field, there will be more lines of force for the coil to cut and hence there will be more emf induced.

- By increasing the speed of the relative motion between the coil and the magnet – If the relative speed between the coil and magnet is increased from its previous value, the coil will cut the lines of flux at a faster rate, so more induced emf would be produced.

## Applications of Faraday’s Law

Faraday law is one of the most basic and important laws of electromagnetism. This law finds its application in most of the electrical machines, industries, and the medical field, etc.

- Power transformers function based on Faraday’s law

- The basic working principle of the electrical generator is Faraday’s law of mutual induction.

- The Induction cooker is the fastest way of cooking. It also works on the principle of mutual induction. When current flows through the coil of copper wire placed below a cooking container, it produces a changing magnetic field. This alternating or changing magnetic field induces an emf and hence the current in the conductive container, and we know that the flow of current always produces heat in it.

- Electromagnetic Flow Meter is used to measure the velocity of certain fluids. When a magnetic field is applied to an electrically insulated pipe in which conducting fluids are flowing, then according to Faraday’s law, an electromotive force is induced in it. This induced emf is proportional to the velocity of fluid flowing.

- Form bases of Electromagnetic theory, Faraday’s idea of lines of force is used in well known Maxwell’s equations. According to Faraday’s law, change in magnetic field gives rise to change in electric field and the converse of this is used in Maxwell’s equations.

- It is also used in musical instruments like an electric guitar, electric violin, etc.

4.2.2 Inductance of a Coil

The inductance of a coil refers to the electrical property the inductive coil has to oppose any change in the current flowing through it. It therfore follows that inductance is only present in an electric circuit when the current is changing.

Inductors generate a self-induced emf within themselves as a result of their changing magnetic field. In an electrical circuit, when the emf is induced in the same circuit in which the current is changing this effect is called Self-induction, (L) but it is sometimes commonly called back-emf as its polarity is in the opposite direction to the applied voltage.

When the emf is induced into an adjacent component situated within the same magnetic field, the emf is said to be induced by Mutual-induction, ( M ) and mutual induction is the basic operating principal of transformers, motors, relays etc. Self inductance is a special case of mutual inductance, and because it is produced within a single isolated circuit we generally call self-inductance simply, Inductance.

The basic unit of measurement for inductance is called the Henry, ( H ) after Joseph Henry, but it also has the units of  Webers per Ampere ( 1 H = 1 Wb/A ).

Lenz’s Law tells us that an induced emf generates a current in a direction which opposes the change in flux which caused the emf in the first place, the principal of action and reaction. Then we can accurately define Inductance as being: “a coil will have an inductance value of one Henry when an emf of one volt is induced in the coil were the current flowing through the said coil changes at a rate of one ampere/second”.

In other words, a coil has an inductance, ( L ) of one Henry, ( 1H ) when the current flowing through the coil changes at a rate of one ampere/second, ( A/s ). This change induces a voltage of one volt, ( VL ) in it. Thus the mathematical representation of the rate of change of current through a wound coil per unit time is given as:

The voltage induced in a coil, ( VL ) with an inductance of L Henries as a result of this change in current is expressed as:

Note that the negative sign indicates that voltage induced opposes the change in current through the coil per unit time ( di/dt ).

From the above equation, the inductance of a coil can therefore be presented as:

Inductance of a Coil

Where: L is the inductance in Henries, VL is the voltage across the coil and di/dt is the rate of change of current in Amperes per second, A/s.

A) Self Inductance

The property of a circuit, by which an e.m.f. is induced in the circuit whenever the current flowing it changes, is termed as Self-Inductance.

Consider a coil of N turns carrying a current of 1A.The flux linking with the coil be Φ webers.

The lines of flux linking the coil change with change in current. This will induce an e.m.f. according to Faraday’s Law. The e.m.f. induced is called self-induced e.m.f.

Inductors are made from individual loops of wire combined to produce a coil and if the number of loops within the coil are increased, then for the same amount of current flowing through the coil, the magnetic flux will also increase.

So by increasing the number of loops or turns within a coil, increases the coils inductance. Then the relationship between self-inductance, ( L ) and the number of turns, ( N ) and for a simple single layered coil can be given as:

### Coefficient of  Self Inductance of a Coil

Where:

- L is in Henries

- N is the Number of Turns

- Φ is the Magnetic Flux

- Ι  is in Amperes

This expression can also be defined as the magnetic flux linkage, ( NΦ ) divided by the current, as effectively the same value of current flows through each turn of the coil. Note that this equation only applies to linear magnetic materials.

## Example 7

A hollow air cored inductor coil consists of 500 turns of copper wire which produces a magnetic flux of 10mWb when passing a DC current of 10 amps. Calculate the self-inductance of the coil in milli-Henries.

Solution

## Example 8

Calculate the value of the self-induced emf produced in the same coil after a time period of 10 milli-seconds (10ms).

Solution

Coefficient of self-inductance for an air cored coil (solenoid)

The self-inductance of a coil or to be more precise, the coefficient of self-inductance also depends upon the characteristics of its construction. For a coil, the magnetic flux that is produced in its inner core is equal to:

Where: Φ is the magnetic flux, B is the flux density, and A is the area.

If the inner core of a long solenoid coil with N number of turns per metre length is hollow, “air cored”, then the magnetic induction within its core will be given as:

Then by substituting these expressions in the first equation above for Inductance will give us:

By cancelling out and grouping together like terms, then the final equation for the coefficient of self-inductance for an air cored coil (solenoid) is given as:

Where:

- L is in Henries

- μο is the Permeability of Free Space (4.π.10-7)

- N is the Number of turns

- A is the Inner Core Area (πr 2) in m2

- ℓ is the length of the Coil in metres

As the inductance of a coil is due to the magnetic flux around it, the stronger the magnetic flux for a given value of current the greater will be the inductance. So a coil of many turns will have a higher inductance value than one of only a few turns and therefore, the equation above will give inductance L as being proportional to the number of turns squared N2.

Example 9  A coil is wound with 500 turns. A current of 5 amperes flowing through the coil produces flux Φ  of 80μ weber. Calculate its inductance.

Solution:

L= 500×80×10-6/5 Henry

L=0.0008 Henry

Example 10 If the current through a coil having an inductance of 0.5H is reduced from 5A to 1A in 0.05 seond, calculate the mean value of the e.m.f. induced in the coil.

Solution: Average rate of change of current

=(1-5)/0.05= -80A/sec.

Average induced e.m.f. in the coil

VL=-0.5(-80)=40V

# B)Mutual Inductance

Mutual Inductance is the interaction of one coils magnetic field on another coil as it induces a voltage in the adjacent coil.

When the emf is induced into an adjacent coil situated within the same magnetic field, the emf is said to be induced magnetically, inductively or by Mutual induction, symbol ( M ). Then when two or more coils are magnetically linked together by a common magnetic flux they are said to have the property of Mutual Inductance.

Mutual Inductance is the basic operating principal of the transformer, motors, generators and any other electrical component that interacts with another magnetic field.

The amount of mutual inductance that links one coil to another depends very much on the relative positioning of the two coils. If one coil is positioned next to the other coil so that their physical distance apart is small, then nearly all of the magnetic flux generated by the first coil will interact with the coil turns of the second coil inducing a relatively large emf and therefore producing a large mutual inductance value.

Likewise, if the two coils are farther apart from each other or at different angles, the amount of induced magnetic flux from the first coil into the second will be weaker producing a much smaller induced emf and therefore a much smaller mutual inductance value. So the effect of mutual inductance is very much dependent upon the relative positions or spacing, ( S ) of the two coils and this is demonstrated below.

### Mutual Inductance between Coils

If the two coils are tightly wound one on top of the other over a common soft iron core unity coupling is said to exist between them as any losses due to the leakage of flux will be extremely small. Then assuming a perfect flux linkage between the two coils the mutual inductance that exists between them can be given as.

- Where:

- µo is the permeability of free space (4.π.10-7)

- µr is the relative permeability of the soft iron core

- N is in the number of coil turns

- A is in the cross-sectional area in m2

- ℓ is the coils length in meters

## Mutual Induction

Here the current flowing in coil one, L1 sets up a magnetic field around itself with some of these magnetic field lines passing through coil two, L2 giving us mutual inductance. Coil one has a current of I1 and N1 turns while, coil two has N2 turns. Therefore, the mutual inductance, M12 of coil two that exists with respect to coil one depends on their position with respect to each other and is given as:

Likewise, the flux linking coil one, L1 when a current flows around coil two, L2 is exactly the same as the flux linking coil two when the same current flows around coil one above, then the mutual inductance of coil one with respect of coil two is defined as M21. Because of this, we can write the mutual inductance between the two coils as: M12 = M21 = M.

The self inductance of each individual coil is given as:

and

By cross-multiplying the two equations above, the mutual inductance, M that exists between the two coils can be expressed in terms of the self inductance of each coil.

giving us a final and more common expression for the mutual inductance between the two coils of:

### Mutual Inductance Between Coils

However, the above equation assumes zero flux leakage and 100% magnetic coupling between the two coils, L1 and L2. In reality there will always be some loss due to leakage and position, so the magnetic coupling between the two coils can never reach or exceed 100%, but can become very close to this value in some special inductive coils.

If some of the total magnetic flux links with the two coils, this amount of flux linkage can be defined as a fraction of the total possible flux linkage between the coils. This fractional value is called the coefficient of coupling and is given the letter k.

## Coupling Coefficient

Generally, the amount of inductive coupling that exists between the two coils is expressed as a fractional number between 0 and 1 instead of a percentage (%) value, where 0 indicates zero or no inductive coupling, and 1 indicating full or maximum inductive coupling.

In other words, if k = 1 the two coils are perfectly coupled, if k > 0.5 the two coils are said to be tightly coupled and if k < 0.5 the two coils are said to be loosely coupled. Then the equation above which assumes a perfect coupling can be modified to take into account this coefficient of coupling, k and is given as:

### Coupling Factor between Coils

or

When the coefficient of coupling, k is equal to 1, (unity) such that all the lines of flux of one coil cuts all of the turns of the second coil, that is the two coils are tightly coupled together, the resulting mutual inductance will be equal to the geometric mean of the two individual inductances of the coils.

Also when the inductances of the two coils are the same and equal, L1 is equal to L2, the mutual inductance that exists between the two coils will equal the value of one single coil as the square root of two equal values is the same as one single value as shown.

More Detail

More Detail

# 4.2.3 Statically and Dynamically Induced EMF

## Induced EMF

When a magnetic flux linking a conductor or coil changes, an electromotive force (EMF) is induced in the conductor or coil, is known as induced EMF. Depending upon the way of bringing the change in magnetic flux, the induced EMF is of two types −

- Statically Induced EMF

- Dynamically Induced EMF

## Statically Induced EMF

When the conductor is stationary and the magnetic field is changing, the induced EMF in such a way is known as statically induced EMF (as in a transformer). It is so called because the EMF is induced in a conductor which is stationary. The statically induced EMF can also be classified into two categories −

- Statically Induced EMF

- Mutually Induced EMF

## Self-Induced EMF

When an EMF is induced in the coil due to the change of its own magnetic flux linked with it is known as self-induced EMF.

Explanation − When a current flows in a coil, a magnetic field produced by this current through the coil. If the current in the coil changes, then the magnetic field linking the coil also changes. Therefore, according to Faraday’s law of electromagnetic induction, an EMF being induced in the coil. The induced EMF in such a way is known as self-induced EMF.

Mathematically, self-induced EMF is given by,

e=Ldi/dt...(1)

Where, L is the self-inductance of the coil.

## Mutually Induced EMF

When an EMF is induced in a coil due to changing magnetic flux of neighbouring coil is known as mutually induced EMF.

Explanation − Consider two coils coil-1 and coil-2 placed adjacent to each other (see the figure). A fraction of the magnetic flux produced by coil-1 links with the coil-2. This magnetic flux which is common to both the coils 1 and 2 is known as mutual flux (φm). Now, if the current in coil-1 changes, the mutual flux also changes and thus EMF being induced in both the coils. The EMF induced in coil-2 is known as mutually induced EMF, since it is induced due changing in flux which is produced by coil-1. Mathematically, the mutually induced EMF is given by,

em=Mdi1/dt...(2)

Where, M is the mutual inductance between the coils.

## Dynamically Induced EMF

When the conductor is moved in a stationary magnetic field so that the magnetic flux linking with it changes in magnitude, as the conductor is subjected to a changing magnetic, therefore an EMF will be induced in it. The EMF induced in this way is known as dynamically induced EMF (as in a DC or AC generator). It is so called because EMF is induced in a conductor which is moving (dynamic).

Explanation − Consider a conductor of length l meters moving with a velocity of v m/s at right angles to a uniform stationary magnetic field of flux density B Wb/m2.Let the conductor moves through a small distance dx in time dt seconds. Then,

Area swept by conductor, a=l×dxm2

∴Magneticfluxcutbyconductor,dψ=MagneticFluxDensity×AreaSwept

⟹dψ=BldxWb

Now, according to Faraday’s law of electromagnetic induction, the induced EMF will be,

e=Ndψ/dt=Bldx/dt(∵N=1)

∵dx/dt=VelocityV

∴e=BlvVolts...		(3)

Equation (3) gives the dynamically induced EMF when the conductor moves at right angle to the magnetic field.

If the conductor moves at an angle &theta to the magnetic field, then the EMF induced due to only the perpendicular component of the velocity to the magnetic field.

e=Blvsinθ...(4)

## Example 11

Two inductors whose self-inductances are given as 75mH and 55mH respectively, are positioned next to each other on a common magnetic core so that 75% of the lines of flux from the first coil are cutting the second coil. Calculate the total mutual inductance that exists between the two coils.

Solution:

## Example 12

When two coils having inductances of 5H and 4H respectively were wound uniformly onto a non-magnetic core, it was found that their mutual inductance was 1.5H. Calculate the coupling coefficient that exists between.

Solution:

Example 13. Two coils have a mutual inductance of 0.6 H. if the current in one coil varied from 4 to 1 A in 0.6 s, calculate :

(a) the emf induced in the second coil

(b) the change of flux linked with the second coil, assuming that it is wound with 200 turns.

Solution:

(a) Average rate of increase of current in one coil= (1-4)/0.6= -5A/s

So the average emf induced in other coil

= -0.6 Henry×(-5)A/S

= 3V Ans

(b) If the change of flux linked with the second coil is ϕ, then emf induced in that coil

= ϕ×number of turns/time in seconds

3= ϕ×200/0.6

So 					ϕ=3×0.6/200

=0.009 Wb=9mWb Ans.

Example 14 A wooden ring has a mean diameter of 150 π mm and a crosss-sectional area of 250 mm2. It is wound with 1500 turns of insulated wire. A second coil of 900 turns is then wound on the top of the first. Assuming that all the flux produced by the first coil links with the second, calculate the mutual inductance.

Solution:

Number of turns on coilA, N1=1500

Number of turns on coilB, N2=900

Relative permeability of wood , µr=1.0

(µo=4.π.×10-7)

Area of cross section of wooden ring

A =250 mm2 =250.×10-4 m2

Length of wooden ring l=150 π mm= 0.15πm

Mutual Inductance =

M= 1500×900 ×4.π.×10-7×1×2.5×10-4/0.15π

M= 0.9mH Ans.

4.3.1 The energy stored in an inductor

The energy stored in an inductance coil having inductance L and R is the resistance of the coil.

Suppose a current I is applied to an inductor or inductance coil of inductance L

such that current through the inductor grows from zero value to a maximum value I. Let current through the inductor at any instant of time t be i.

## Let i be the instantaneous current in the circuit then applying Kirchoff's voltage law, we get

## E = iR + Ldi/dt

## The power supplied by the battery is given by

## P = Ei = i2R + Li di/dt

## where i2R is the power dissipated in the resistor and the last term represents the rate at which energy is being supplied to the inductor.

## A emf induced in the inductor which opposes the flow of current and is given by the formula,

## E=−Ldi/dt

## where L is the inductance and di/dt is the rate of change of current.

## In order to pass current through the current, work must be done by the voltage source against this emf. The formula for rate of work done is given by,

## dW*dt=−Ei

## ⇒dW=−Eidt

## Putting the value ofE, we get,

## dW=−(−Ldi/dt)idt

## ⇒dW=iLdi

The energy stored in the inductor can be calculated as

When current in the coil changes from zero to I amperes, emf induced in it is equal to

L times rate of change of current.

Time taken by the current from zero to I ampere is t seconds.

Then 	E= - L(0_I)/t    = IL/t volts

Average current         =(0+I)/2  =I/2

Energy stored          = {current × e.m.f.× time }= I/2 × (IL/t)  × t =0.5 LI2

Note: Remember, one function of an inductor is to store electrical energy. There is one more component called capacitor. A capacitor stores energy in the electric field whereas an inductor stores energy in the magnetic field, students sometimes get confused between these two components.

The battery that establishes the current in an inductor has to do work against the opposing induced emf. The energy  supplied by the battery is stored in the inductor.

Example 15 The field winding of electromagnet of d.c. motor has 1500 turns and 110ohm resistance. Its exciting voltage is 220V, which produce a flux of 6mWb. Find out the energy stored in the field of motor.

Solution.

Energy stored in the magnetic circuit=0.5 LI2

Where

Current I= 220/110    = 2 A

L= 0.5(1500×6×10-3) = 3 Henry

Energy stored W= 0.5 LI2

W= 0.5×3×4  = 6 Joules.

# 4.3.2Series and Parallel Combination of Inductors:

# A)Inductors in Series

Inductors can be connected together in a series connection when the are daisy chained together sharing a common electrical current

These interconnections of inductors produce more complex networks whose overall inductance is a combination of the individual inductors. However, there are certain rules for connecting inductors in series or parallel and these are based on the fact that no mutual inductance or magnetic coupling exists between the individual inductors.

Inductors are said to be connected in “Series” when they are daisy chained together in a straight line, end to end. In the Resistors in Series tutorial we saw that the different values of the resistances connected together in series just “add” together and this is also true of inductance. Inductors in series are simply “added together” because the number of coil turns is effectively increased, with the total circuit inductance LT being equal to the sum of all the individual inductances added together.

### Inductor in Series Circuit

The current, ( I ) that flows through the first inductor, L1 has no other way to go but pass through the second inductor and the third and so on. Then, series inductors have a Common Current flowing through them, for example:

IL1 = IL2 = IL3 = IAB …etc.

In the example above, the inductors L1, L2 and L3 are all connected together in series between points A and B. The sum of the individual voltage drops across each inductor can be found using Kirchoff’s Voltage Law (KVL) where, VT = V1 + V2 + V3 and we know from the previous tutorials on inductance that the self-induced emf across an inductor is given as: V = L di/dt.

So by taking the values of the individual voltage drops across each inductor in our example above, the total inductance for the series combination is given as:

By dividing through the above equation by di/dt we can reduce it to give a final expression for calculating the total inductance of a circuit when connecting inductors together in series and this is given as:

### Inductors in Series Equation

### Ltotal= L1+L2+L3+…….+ Ln

Then the total inductance of the series chain can be found by simply adding together the individual inductances of the inductors in series just like adding together resistors in series. However, the above equation only holds true when there is “NO” mutual inductance or magnetic coupling between two or more of the inductors, (they are magnetically isolated from each other).

One important point to remember about inductors in series circuits, the total inductance ( LT ) of any two or more inductors connected together in series will always be GREATER than the value of the largest inductor in the series chain.

## Example 16

Three inductors of 10mH, 40mH and 50mH are connected together in a series combination with no mutual inductance between them. Calculate the total inductance of the series combination.

## Mutually Connected Inductors in Series

When inductors are connected together in series so that the magnetic field of one links with the other, the effect of mutual inductance either increases or decreases the total inductance depending upon the amount of magnetic coupling. The effect of this mutual inductance depends upon the distance apart of the coils and their orientation to each other.

Mutually connected series inductors can be classed as either “Aiding” or “Opposing” the total inductance. If the magnetic flux produced by the current flows through the coils in the same direction then the coils are said to be Cumulatively Coupled. If the current flows through the coils in opposite directions then the coils are said to be Differentially Coupled as shown below.

### Cumulatively Coupled Series Inductors

While the current flowing between points A and D through the two cumulatively coupled coils is in the same direction, the equation above for the voltage drops across each of the coils needs to be modified to take into account the interaction between the two coils due to the effect of mutual inductance. The self inductance of each individual coil, L1 and L2 respectively will be the same as before but with the addition of M denoting the mutual inductance.

Then the total emf induced into the cumulatively coupled coils is given as:

Where: 2M represents the influence of coil L1 on L2 and likewise coil L2 on L1.

By dividing through the above equation by di/dt we can reduce it to give a final expression for calculating the total inductance of a circuit when the inductors are cumulatively connected and this is given as:

Ltotal=L1+L2+2M

If one of the coils is reversed so that the same current flows through each coil but in opposite directions, the mutual inductance, M that exists between the two coils will have a cancelling effect on each coil as shown below.

### Differentially Coupled Series Inductors

The emf that is induced into coil 1 by the effect of the mutual inductance of coil two is in opposition to the self-induced emf in coil one as now the same current passes through each coil in opposite directions. To take account of this cancelling effect a minus sign is used with M when the magnetic field of the two coils are differentially connected giving us the final equation for calculating the total inductance of a circuit when the inductors are differentially connected as:

Ltotal= L1+ L2- 2M

Then the final equation for inductively coupled inductors in series is given as:

Coefficient of Coupling

The fractional part of magnetic flux produced by the current of one coil which links with the other coil is called co-efficient of coupling . It is denoted by K.

K=M/(√L1 L2)

Where K= co-efficient of coupling between two coils

M= Mutual Inductance between two coils

L1 = Self Inductance of one coil

L2= Self Inductance of other coil

## Example 17

Two inductors of 10mH respectively are connected together in a series combination so that their magnetic fields aid each other giving cumulative coupling. Their mutual inductance is given as 5mH. Calculate the total inductance of the series combination.Also compute the coefficient of coupling.

K=M/(√L1 L2)

= 5/(√10*10)

K=0.5 Ans

## Example 18

Two coils connected in series have a self-inductance of 20mH and 60mH respectively. The total inductance of the combination was found to be 100mH. Determine the amount of mutual inductance that exists between the two coils assuming that they are aiding each other.

# B)Inductors in Parallel

Inductors are said to be connected together in Parallel when both of their terminals are respectively connected to each terminal of another inductor or inductors

The voltage drop across all of the inductors in parallel will be the same. Then, Inductors in Parallel have a Common Voltage across them and in our example below the voltage across the inductors is given as:

VL1 = VL2 = VL3 = VAB …etc

In the following circuit the inductors L1, L2 and L3 are all connected together in parallel between the two points A and B.

### Inductors in Parallel Circuit

In series inductors l, The total inductance, LT of the circuit was equal to the sum of all the individual inductors added together. For parallel connected inductors, the equivalent circuit inductance LT is calculated differently.

The sum of the individual currents flowing through each inductor can be found using Kirchoff’s Current Law (KCL) where, IT = I1 + I2 + I3 and we know from the previous tutorials on inductance that the self-induced emf across an inductor is given as: V = L di/dt

Then by taking the values of the individual currents flowing through each inductor in our circuit above, and substituting the current i for i1 + i2 + i3 the voltage across the parallel combination is given as:

By substituting di/dt in the above equation with v/L gives:

We can reduce it to give a final expression for calculating the total inductance of a parallel circuit, and this is given as:

### Parallel Inductor Equation

Here, like the calculations for parallel resistors, the reciprocal ( 1/Ln ) value of the individual inductances are all added together instead of the inductances themselves. But again as with series connected inductances, the above equation only holds true when there is “NO” mutual inductance or magnetic coupling between two or more of the inductors, (they are magnetically isolated from each other). Where there is coupling between coils, the total inductance is also affected by the amount of coupling.

This method of calculation can be used for calculating any number of individual inductances connected together within a single parallel network. If however, there are only two individual parallel connected inductances then a much simpler and quicker formula can be used to find the total inductance value, and this is:

One important point to remember about parallel inductive circuits, the total inductance ( LT ) of any two or more inductors connected together in parallel will always be LESS than the value of the smallest inductance in the parallel branch.

## Example 19

Three inductors of 60mH, 120mH and 75mH respectively, are connected together in a parallel combination with no mutual inductance between them. Calculate the total inductance of the parallel combination in millihenries.

## Mutually Coupled Parallel Inductors

When inductors are connected together in parallel so that the magnetic field of one links with the other, the effect of mutual inductance either increases or decreases the total inductance depending upon the amount of magnetic coupling that exists between the coils. The effect of this mutual inductance depends upon the distance apart of the coils and their orientation to each other.

Mutually connected parallel inductors can be classed as either “aiding” or “opposing” the total inductance with parallel aiding connected coils increasing the total equivalent inductance and parallel opposing coils decreasing the total equivalent inductance compared to coils that have zero mutual inductance.

Mutual coupled parallel coils can be shown as either connected in an aiding or opposing configuration by the use of polarity dots or polarity markers as shown below.

### Parallel Aiding Inductors

The voltage across the two parallel aiding inductors above must be equal since they are in parallel so the two currents, i1 and i2 must vary so that the voltage across them stays the same. Then the total inductance, LT for two parallel aiding inductors is given as:

Where: 2M represents the influence of coil L 1 on L 2 and likewise coil L 2 on L 1.

If the two inductances are equal and the magnetic coupling is perfect such as in a toroidal circuit, then the equivalent inductance of the two inductors in parallel is L as LT = L1 = L2 = M. However, if the mutual inductance between them is zero, the equivalent inductance would be L ÷ 2 the same as for two self-induced inductors.

If one of the two coils was reversed with respect to the other, we would then have two parallel opposing inductors and the mutual inductance, M that exists between the two coils will have a cancelling effect on each coil instead of an aiding effect as shown below.

### Parallel Opposing Inductors

Then the total inductance, LT for two parallel opposing inductors is given as:

This time, if the two inductances are equal in value and the magnetic coupling is perfect between them, the equivalent inductance and also the self-induced emf across the inductors will be zero as the two inductors cancel each other out.

This is because as the two currents, i1 and i2 flow through each inductor in turn the total mutual flux generated between them is zero because the two flux’s produced by each inductor are both equal in magnitude but in opposite directions.

Then the two coils effectively become a short circuit to the flow of current in the circuit so the equivalent inductance, LT becomes equal to ( L ± M ) ÷ 2.

## Example 20

Two inductors whose self-inductances are of 75mH and 55mH respectively are connected together in parallel aiding. Their mutual inductance is given as 22.5mH. Calculate the total inductance.

## Example 21

Calculate the equivalent inductance of the following inductive circuit.

Calculate the first inductor branch LA, (Inductor L5 parallel with inductors L6 and L7)

Calculate the second inductor branch LB, (Inductor L3 in parallel with inductors L4 and LA)

Calculate the equivalent circuit inductance LEQ, (Inductor L1 in parallel with inductors L2 and LB)

Example 22.

Consider a solenoid with 500 turns which are wound on an iron core whose relative permeability is 800. 40 cm is the length of the solenoid, while 3 cm is the radius. The change in current is from 0 to 3 A. Calculate the average emf induced for this change in the current for a time of 0.4 seconds.

Solution:

Given:

No.of turns, N = 500 turns

Relative permeability, μr = 800

Length, l = 40 cm = 0.4 m

Radius, r = 3 cm = 0.03 m

Change in current, di = 3 – 0 = 3 A

Change in time, dt = 0.4 sec

Self-inductance is given as

L = μN2Al = μ0μrN2𝜋r2/l

Substituting the values we get

(4)(3.14)(10-7)(800)(5002)(3.14)(3×10-2)2/0.4

L = 1.77 H

Magnitude of induced emf, VL = L di/dt = 1.77×3/0.4

VL = 13.275 V

Example 23.

There are two coils such that the current flowing through the first coil experiences a change in current flow from 2 A to 10 A in 0.4 sec. Calculate mutual inductance between the two coils when 60 mV emf is induced in the second coil. Determine the induced emf in the second coil if the current changes from 4 A to 16 A in 0.03 sec in the first coil.

Solution:

Given:

Case 1:

Change in current, di = 10 -2 = 8 A

Change in time, dt = 0.4 sec

Magnitude of induced emf, V2 = 60×10-3 V

Case 2:

Change in current, di = 16 – 4 = 12 A

Change in time, dt = 0.03 sec

Mutual inductance of the second coil with respect to the first coil is given as:

M21 = V2/(di/dt) = 60×10-3×0.4/8 = 3×10-3H

Induced emf in the second coil due to change in the rate of current in the first coil is given as:

V2 = M21 di/dt = 3×10-3×12/0.03 = 1.2V

### Difference between Self and Mutual Inductance

| Self inductance | Mutual inductance |
| --- | --- |
| In self inductance, the change in the strength of current in the coil is opposed by the coil itself by inducing an e.m.f. | In mutual inductance out of the two coils one coil opposes change in the strength of the current flowing in the other coil. |
| The induced current opposes the growth of current in the coil when the main current in the coil increases. | The induced current developed in the neighboring coil opposes the growth of current in the coil when the main current in the coil increases. |
| The induced current opposes the decay of current in the coil when the main current in the coil decreases. | The induced current developed in the neighboring coil opposes the decay of the current in the coil when the main current in the coil decreases. |

### Summary

Concept of Electric Charge

Electric Charge is the fundamental property of matter that develop electrical influence. Electric charge can be positive or negative, stationary or moving one.  Electric charge is the that property of matter due to which it exhibit electric, magnetic and electromagnetic effects.

Coulomb’s Law:

Coulom’s law states that “the magnitude of the electric force between two charges is directly proportional to the product of the two charges and inversely proportional to the square of the distance between them.”

Hence 1 coulomb is that charge when placed at a distance of 1from equal and similar charge, repels it with a force of 9×109Newtons.

The branch of science associated with the study of effects of stationary charges is called electrostatics.

Electromagnetism

A force of attraction  or repulsion that acts  at a distance and caused due to a magnetic field is called magnetism.

The branch of science concerned with the forces that occur between electricity charged particles is called electromagnetism.

Two aspects of electromagnetism are:

i)  Electrcity

ii) Magnetism

The magnetic field is a force created by moving electric charges and magnetic dipoles. It is a vector field. The magnetic force is force of attraction or repulsion between electrically charged particles because of their motion.

Permanent magnets have the property of retaining magnetism indefinitely and they require no electric aid to magnetize like existing coils (ampere turns).

Ampere Turns

Ampere turns is the product of current I flowing through a coil in amperes and the number of turns of coil.

It is represented by AT. It is the magneto motive force (m.m.f.) produced by the coil.

Artificial magnets are made by process of magnetization. A soft iron piece if similarly magnetized will remain its magnetism only for a small time.

Magnetic Material

Substances which are attracted by a magnet are called magnetic materials. Iron, steel and some of their alloys are superior to all other metals. Cobalt, nickel and some of their alloys possess certain magnetic properties.

## Magneticfield

The region around the magnet where the force on the magnetic needle occurs and the needle stops at a specific direction, is called a magnetic field.

These magnetic lines of force have following properties:

1. Magnetic line of forces always starts from North Pole and end at South Pole of the magnet.

2. These lines of forces never intersect each other.

3. Near the poles magnetic lines are very near to each other which shows that magnetic field at the poles is stronger as compare to other parts.

4. They seek the path of least resistance between opposite magnetic poles. In a single bar magnet as shown to the right, they attempt to form closed loops from pole to pole.

5. They all have the same strength.

6.Their density decreases (they spread out) when they move from an area of higher permeability to an area of lower permeability

# Magnetic Flux

Magnetic flux is a measurement of the total magnetic field which passes through a given area. It is a useful tool for helping describe the effects of the magnetic force on something occupying a given area.

The magnetic flux is,

ΦB=BAcosθ

### Magnetic Flux Density

The symbol for magnetic flux density is B and the unit of magnetic flux density is the Tesla, T.

It is important to remember that all calculations for flux density are done in the same units, e.g., flux in webers, area in m2 and flux density in Teslas.

ELECTROMAGNETISM

Electromagnetism is the science that deals with the phenomena associated with electric and magnetic fields and their interaction with each other, with electric charges and currents.

Electromagnet are widely used as a components in electrical devices such as motor, generator, electrical bells MRI machine etc.

The principle of the magnetic effect of electric current used in many appliances like motor etc.

### Magnetic field Due to a Current Carrying Conductor

A current carrying straight conductor has magnetic field in the form of concentric circles around it. Magnetic field of current carrying straight conductor can be shown by magnetic field lines.

The magnetic field of an infinitely long straight wire can be obtained by applying Ampere's law. The expression for the magnetic field is

Where

I is the current

r is the radial distance

The permeability of free space is

The direction of magnetic field through a current carrying conductor depends upon the direction of flow of electric current. The direction of magnetic field gets reversed in case of a change in the direction of electric current.

The direction of magnetic field, in relation to direction of electric current through a straight conductor can be depicted by using the Right Hand Thumb Rule. It is also known as Maxwell’s Corkscrew Rule.

If a current carrying conductor is held by right hand, keeping the thumb straight and if the direction of electric current is in the direction of thumb, then the direction of wrapping of other fingers will show the direction of magnetic field.

The magnitude of magnetic field is directly proportional to magnitude of current flowing through the coil and is inversely proportional to the radius of coil.

B α I

B α1/r

The formula for the field inside the solenoid is

B = I N / L

Where, B is the magnetic field

I is the current in the coil

N is the number of turns in the solenoid

L is the length of the coil

Total flux of solenoid:

In a long solenoid the flux density is uniform over the entire cross sectional area A so that the total flux

ϕ= B.A =(I N / L).A                              ---(in air)

ϕ= B.A =(I N / L).A                              ---(in medium)

### (a) Flux density (B):

The magnetic flux lines start and end in such a way that they form closed loops. Weber (Wb) is the unit of magnetic flux (Φ). Flux density (B) is the flux per unit area. Tesla (T) or Wb/m2 is the unit of flux density.

where B is a quantity called magnetic flux density in teslas, Φ is the total flux in webers and A is the area perpendicular to the lines in m2.

### (b) Magneto-motive force MMF (ζ):

A measure of the ability of a coil to produce a flux is called the magneto-motive force. It may be considered as a magnetic pressure, just as emf is considered as an electric pressure. A coil with N turns which is carrying a current of I amperes constitutes a magnetic circuit and produces an mmf of NI ampere turns. The source of flux (Φ) in the magnetic circuit is the mmf. The flux produced in the circuit depends on mmf and the length of the circuit.

### (c) Magnetic field strength / Field Strength(H):

The magnetic field strength of a circuit is given by the mmf per unit length.

The magnetic flux density (B) and its intensity (field strength) in a medium can be related by the following equation

where

- μ = μ0μr is the permeability of the medium in Henrys/metre (H/m),

- μ0 = absolute permeability of free space and is equal to 4π x 10-7 H/m and

- μr = relative permeability of the medium.

Relative permeability is a non-dimensional numeric which indicates the degree to which the medium is a better conductor of magnetic flux as compared to free space. The value of μr = 1 for air and non-magnetic materials. It varies from 1,000 to 10,000 for some types of ferro-magnetic materials.

### (d) Reluctance (R):

It is the property of the medium which opposes the passage of magnetic flux. The magnetic reluctance is analogous to resistance in the electric circuit. Its unit is AT/Wb. Air has a much higher reluctance than does iron or steel. For this reason, Magnetic Circuit Analysis used in electrical machines are designed with very small air gaps.

According to definition, reluctance = mmf/flux

The reciprocal of reluctance is known as permeance 1/R = Φ/ζ

Thus reluctance is a measure of the opposition offered by a magnetic circuit to the, setting up of the flux. The reluctance of the magnetic circuit is given by R = 1/μ 1/a.

where I is the length, a is the cross-sectional area of the Magnetic Circuit Analysis and it is the permeability of the medium.

From the above equations

Permeance(P)

It is reciprocal of reluctance and it is the readiness with which magnetic flux is developed. It is analogous to conductance in an electric circuit. Its unit is weber per ampere turn.

Thus P=Φ/ζ    ( Flux  /m.m.f.  )

Analogy between Magnetic and Electric Circuits:

Similarities:Similarities between electrical and magnetic circuits are given below.

| Magnetic circuit | Electric circuit |
| --- | --- |
| The close path for the magnetic flux is called magnetic circuit. | The close path for the electric current is called electric circuit. |
| The number of magnetic lines of force decide the magnetic flux. | Flow of electron decide the current passing through the conductor. |
| Flux (ф) = MMF/Reluctance | Current (I) = EMF/Resistance |
| MMF is the driving force in the magnetic circuit. The unit of MMF is ampere-turns (AT). | EMF is the driving force in the electric circuit. The unit of EMF is volts (V). |
| Reluctance (S) opposed by the magnetic path to the flux. The unit of reluctance is AT/wb. | Resistance (R) oppose the flow of current. The unit of resistance is ohm (Ω). |
| Flux (ф) measured in weber (Wb). | Current (I) measured in Ampere (A). |
| MMF measured in Amp Turns (AT). | EMF measured in Volts (V). |
| Permanence = 1/Reluctance = 1/S | Conductance = 1/Resistance = 1/R |
| Permeability (μ) | Conductivity (σ) |
| Reluctivity | Resistivity |
| Flux Density (B) = ф/A (Wb/m2) | Current Density (J) = I / A (A/m2) |
| Magnetic Intensity (H) = NI/L (AT/m) | Electric Intensity (E) = V/d (V/m) |
| Kirchhoff MMF Law and Flux Law is applicable to the Magnetic Flux. | Kirchhoff Current Law (KCL) and Kirchhoff Voltage Law (KVL) is applicable to the electrical circuit. |

### Dissimilarities:There are few dissimilarities between the two circuits which are listed below:

| Magnetic circuit | Electric circuit |
| --- | --- |
| There is no magnetic insulator as flux can pass through all the materials, even through the air as well. For magnetic flux there is no perfect insulators. | There are many materials which used as insulators. From which current cannot pass. i.e. air, PVC, glass, synthetic resin etc. |
| Magnetic flux does not flow but it set up in the magnetic circuit. | The electric current actually flows in an electric circuit. |
| At constant temperature, the reluctance (S) of a magnetic circuit is not constant but varies with μr. | At constant temperature, the resistance of an electric circuit is constant as its value depends on resistivity which is almost constant. |
| Once magnetic flux is setup in a magnetic circuit, no energy needed. | Energy needed as long as current flows through electric circuit. |

Ampere Turns:

Ampere turns is the product of current I flowing through a coil in amperes and the number of turns of coil. It is represented by AT.It is the m.m.f. produced by the coil.

## Faraday’s Laws of Electromagnetic Induction

Faraday’s Laws of Electromagnetic Induction consists of two laws. The first law describes the induction of emf in a conductor and the second law quantifies the emf produced in the conductor.

### Faraday's Second Law:

Faraday's second law of electromagnetic induction states that,  the magnitude of induced emf is equal to the rate of change of flux linkages with the coil. The flux linkages is the product of number of turns and the flux associated with the coil.

### Lenz's Law

Lenz's  law of electromagnetic induction states that, when an emf is induced according to Faraday's law, the polarity (direction) of that induced emf is such that it opposes the cause of its production.

Thus, considering Lenz's law

E = -N (dΦ/dt)   (volts)

The negative sign shows that, the direction of the induced emf and the direction of change in magnetic fields have opposite signs.

Types of Induced emf

An Electromotive Force or EMF is said to be induced when the flux linking with a conductor or coil changes.

This change in flux can be obtained in two different ways; that is by statically or by dynamically induced emf.

- Statically Induced Electromotive Force

- Dynamically Induced Electromotive Force

## 1. STATICALLY INDUCED EMF

This type of EMF is generated by keeping the coil and the magnetic field system, stationary at the same time; that means the change in flux linking with the coil takes place without either moving the conductor (coil) or the field system.

This change of flux produced by the field system linking with the coil is obtained by changing the electric current in the field system.

It is further divided in two ways

- Self-induced electromotive force (emf which is induced in the coil due to the change of flux produced by it linking with its own turns.)

- Mutually induced electromotive force(emf which is induced in the coil due to the change of flux produced by another coil, linking with it.)

## 2. DYNAMICALLY INDUCED EMF

In dynamically induced electromotive force the magnetic field system is kept stationary, and the conductor is moving, or the magnetic field system is moving, and the conductor is stationary. Thus by following either of the two process the conductor cuts across the magnetic field and the emf is induced in the coil.

This phenomenon takes place in electric generators and back emf of motors and also in transformers.

Self-inductance finds its applications in the following examples:

- Tuning circuits

- Inductors that are used as relays

- Various sensors

- Induct the ion motors and transformers

Mutual inductance finds its applications in the following examples:

- Transformers use the concept to alter one AC voltage to another

- Mutual inductance finds its use in pacemakers (used for heart patients)

- Digital signal processing is an example wherein mutual inductance is lowered by the counter-winding of coils

- An electric cloth dryer uses coils for heating

- The coils can be wound in a counter manner such that their combined magnetic fields balance out by cancelling each other, significantly reducing the mutual inductance

- Metal detectors at airports also make use of this concept

### Derivation of Inductance

Consider a DC source. When the switch is turned on, the current flows from zero to a certain value such that there is a change in the rate of current flowing. Let φ be the change in flux due to current flow. The change in flux is with respect to time which is given as:   dφ/dt

Apply Faraday’s law of electromagnetic induction,

E=Ndϕ/dt

Where,

- N is the number of turns in the coil

- E is the induced EMF across the coil

From Lenz’s law, we can write the above equation as

E=−Ndϕ/dt

The above equation is modified for calculating the value of inductance

E=−Ndϕ/dt

E=−Ldi/dt

N = dΦ = L di

NΦ = Li

Therefore,

Li = NΦ = NBA

Where B is the flux density, A is the area of the coil.

Hl = Ni

Where H is the magnetizing force due to magnetic flux

B = μH

Li = NBA

L = NBA/i = N2BA/Ni

N2BA/Hl = N2μHA/Hl

L = μN2A/l = μN2𝜋r2/l

Where,

- r is the radius of the coil

### What is Inductance?

Inductance is the tendency of an electrical conductor to oppose a change in the electric current flowing through it. It is denoted by L.

### Define one henry.

One henry is defined as the amount of inductance required to produce an emf of 1 volt in a conductor when the current change in the conductor is at the rate of 1 Ampere per second.

### What are the factors that affect inductance?

The following factors affect the inductance in a circuit:

- Number of Wire Turns in the Coil

- Coil Area

- Core Material

- Coil Length

### How is the mutual inductance of a pair affected when the separation between the coils is increased?

When the separation between the coils is increased, the magnetic flux linked with the secondary coil will decrease. Therefore, the mutual inductance of the pair of coils decreases.

### What is the application of mutual inductance?

Mutual inductance can be used in transformers, generators and electric motors.

Q1. How do energy stored in an inductor?

Ans. Let assume we have an electrical circuit containing a power source and a solenoid for inductance L, we can write the magnetic field number, E, stored in the inductor as E = 1/2 x L x I², where I is the current flowing through the wire.

Q2. How much energy is stored in an inductor in a steady-state?

Ans. If the current flowing in the inductor does not change as in the DC circuit, then there will be no change in the stored energy, such as P = Li (di / dt) = 0.

Q3. What is the inductor formula?

Ans. We know that the voltage across an inductor is given by the equation. V = L di / dt.

We can write, vAB = LTotal x dlt / dt. VAB = LTotal x d (I1 + I2 + I3) / dt.

Q4. What is the energy stored in the capacitor?

Ans. Electrostatic potential energy gets stored in the capacitor. It is then related to the charge and voltage between the capacitor plates.

Q5. What is the unit of inductor?

Ans. The SI unit of inductance is Henry, abbreviated as 'H'. It is defined as the measure of electric current changes at one ampere per second, resulting in an electromotive force of one volt across the inductor.

What do we call the phenomenon of production of back emf in a coil due to the flow of varying current through it?

- Self-inductance

- Electromagnetic Induction

- Magnetic flux

- Magnetic moment

Answer: A. Self-inductance is that phenomenon in which charge in electric current in a coil produces an induced emf in the coil itself.

Q2. Two coils A and B have L= 2×10-2  Henry. If the current in the primary is i = 5 sin 10πθ  then maximum value of emf induced in coil B is:

- Π volt

- Π2 volt

- Π3 volt

- Π4 volt

Answer: A. Given that, Current i = 5sin (10πt), Mutual inductance L = 2×10-2 and H =0.02H

Induced emf, E = -M didt

∴ |E| = M × 5 (10π) cos (10πt)

= π volt

Multiple Choice Questions:

  - Molecular theory of magnetism is given by :

    - Enwine	(b) Weber

(c) Faraday	(d) Fleming.

  - The value of permeability of free space is

    - 4 × 10–9 henry/metre	(b) 4 × 10–8 henry/metre

(c) 4 × 10–7 henry/metre	(d) 4 × 10–6 henry/metre

  - A solenoid coil must have

    - Length and breadth equal	(b) Length greater than breadth

(c) Breadth greater than length	(d) None of the above.

  - In the core of a solenoid

    - Copper is used	(b) Soft iron is used

(c) Aluminium is used	(d) None of the above.

  - The property of a material which opposes the creation of magnetic flux in it is known as

    - Magnetomotive force	(b) Reluctance

(c) Permeance	(d) Reluctivity.

  - Reciprocal of reluctance is

    - Susceptibility	(b) Permeability

(c) Permeance	(d) Reluctivity.

  - The force between two long parallel conductors is inversely proportional to

    - Current in one conductor	(b) Product of current in two conductors

(c) Distance between the conductors	(d) Radius of conductors.

  - The direction magnetic lines of force is determined by

    - Right hand rule	(b) Right hand cork screw rule

- Both of the above rules	(d) None of the above rules.

ELECTROSTATICS AND MAGNETOSTATICS

3-27

  - The direction of magnetic lines of force, when current is into the plane of paper, is in

    - Clockwise direction	(b) Anti clockwise direction

(c) In a straight line	(d) None of the above.

  - The direction magnetic lines of force, when current is out of the plane of paper is in

    - Clockwise direction	(b) Anti clockwise direction

(c) In a straight line	(d) None of the above.

  - When the magnetic lines of force are in clockwise direction the direction of current is marked by a

    - Core	(b) Dot

(c) Blank circle	(d) None of the above.

  - A dot in the circle indicates current flowing

    - Into the plane of paper	(b) Out of the plane of paper

(c) In clockwise direction	(d) In anti clockwise direction.

  - When the direction of current in a solenoid coil is anti clockwise the polarity at the nearer end will be

    - North	(b) South

(c) North South both	(d) None of the above.

  - When the direction of current in a solenoid coil is clockwise, the polarity at the farther end will be

    - North pole	(b) South pole

(c) North and South pole	(d) None of the above.

  - When a current carrying conductor is placed in magnetic field it experiences

    - Increase in e.m.f.	(b) Decrease in e.m.f.

(c) A force	(d) None of the above.

  - Force end a current carrying conductor in magnetic field is measured in

    - Newtons	(b) Newton-metres

(c) Kilogram-metres	(d) None of the above.

  - Direction of force on a conductor in a magnetic field is determined by

    - Gleming’s right hand rule	(b) Fleming’s left hand rules

(c) Right hand thumb rule	(d) None of the above.

  - According to Fleming’s left hand rule the direction of force is indicated by

    - Middle finger	(b) First finger

(c) Thumb	(d) Encircling of fingers.

  - The unit of reluctance is :

    - Henry	(b) (Henry) –1

(c) Henry/metre	(d) Metre/henry.

  - Reciprocal of reluctance is

    - Susceptibility	(b) Permeability

(c) Permeance	(d) Reluctivity.

  - The unit of reluctance is

    - Ampere turn	(b) Ampere turn/metre

(c) Ampere turn/weber	(d) It is dimensionless.

  - While comparing magnetic and electric circuits, the flux of magnetic circuit is compared with, which parameter of electric circuit

    - Current	(b) reluctance

(c) Current density	(d) e.m.f.

  - While comparing magnetic and electric circuit, the point of dissimilarity exists while considering

    - m.m.f. and em.f.	(b) Reluctance and resistance

(c) Flux and current flow	(d) Permeance and conductance.

  - The magnetomotive force of magnetic circuit is analogous to which quantity of electric circuit

    - e.m.f.	(b) Current

(c) Volt ampere	(d) Power.

  - The relative permeability is less than unity in case of

    - Non ferrous materials	(b) Ferrites

(c) Dimagnetic materials	(d) Ferro magnetic materials.

  - Reluctivity is analogous to

    - Permeability	(b) Conductivity

(c) Resistivity	(d) None of the above.

    - Charge on electron is

  - -1.6×10-19F

  - -1.6×10-19C

  - -1.6×10-19N

  - Zero

28 	Charge on an atom is

- Always negative

- -1.6×10-19C

- Always negative

- Zero

29	 Unit of magneto motive force is

  - Weber

  - Tesla

  - Amp-turns

  - Henry

30	Unit of flux  density is

  - Weber/m2

  - Tesla

  - Joule

  - Weber/m

31	Energy stored in a magnetic field is measured in

- kWh

- Coulombs

- Joules

- Watts

32	EMF induced in a coil is directly proportional to

- rate of change of current

- rate of change of flux

- both a and b

- a or c

33	The emf induced in a coil depends on

- Number of turns

- change of flux

- time taken

- all of the above

34	Direction of induced emf is found by

- Fleming’s Left hand rule

- Fleming’s Right hand rule

- Lenz’s Law

- Both b and c

35 . Magnetic effect of current was discovered by
a Oersted
b Faraday
c Bohr
d Ampere

| ANSWERS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. (b) | 2. (c) | 3. | (b) | 4. (b) | 5. (b) |
| 6. (c) | 7. (c) | 8. | (c) | 9. (a) | 10. (b) |
| 11. (a) | 12. (b) | 13. | (b) | 14. (b) | 15. (c) |
| 16. (a) | 17. (b) | 18. | (c) | 19. (b) | 20. (c) |
| 21. (c) | 22. (a) | 23. | (c) | 24. (c) | 25. (c) |
| 26. (c) | 27 (b) | 28(d) | 29(c) | 30(a) | 31 (c) |
| 32(c) | 33(d) | 34(b) | 35(a) |  |  |

Fill in the Blanks

    - Electric charge ………….be conserved.

    - Electric charge is a ……….quantity.

    - Magnetic lines of force……interact with each other.

    - Magnetic flux can be measured by a….. meter.

    - The reluctance of a magnetic circuit is similar to …….of an electric circuit.

    - The emf induced by the inductor is also known as …….emf.

    - In a magnetic circuit energy is required to …… an magnetic field.

    - Magnetic lines of force are …..at the pole.

    - In a series magnetic circuitL= ……

    - Self induced emf in a coil =…….

    - Co-efficient of self inductance is flux linkage per….

    - Inductance of a coil depends upon …..of turns of coil.

    - Coefficient of mutual induction M=……

    - Faradays first law of electromagnetic induction….explain magnitude of induced e.m.f.=……………

    - ………………is the opposition offered to the flow of magnetic flux in a magnetic circuit.

Answers:

- Can 2. Scalar 3.  do not  4. Flux 5. Resistance 6. Back  7. create 8.  Crowded

9.  L1+L2+…..  10.  L*di/dt  11. Ampere  12. Number  13. N2Φ2/I1  14. Does not 15. Reluctance

- VERY SHORT ANSWER TYPE QUESTIONS

  - Name the two type of charges in electrostatics.

  - What is the unit of electric charge.

  - How many poles exists in a magnet ? Name them.

  - Which rule is used to find direction of current in a conductor moving in magnetic field?

  - Reluctance is analogous to which quantity in electric circuit.

  - Current is analogous to which quantity in magnetic circuit.

  - Which rule is used to find the direction of Force in a conductor moving in magnetic field.

  - What is the unit of m.m.f.

  - Write the unit of magnetic flux density.

  - What is the permeability value in air.

  - Write the expression showing relationship between magnetic flux density, permeability and magnetic field intensity.

  - Write the unit of reluctance.

  - Write expression for the force an current carying conductor in a magnetic field.

  - Write expression for the total flux due to solenoid.

  - How do you find the direction of magnetic field around a current carrying conductor.

- ◆ ◆ PRINCIPLES OF ELECTRICAL ENGINEERING

ELECTROSTATICS AND MAGNETOSTATICS

16. Fringing phenomenon is found in which portion of magnetic circuit.

3-29

- Define reluctance.	[HSBTE, Dec. 2018]

- Define MMF.	[HSBTE, Dec. 2018]

- Define flux density and write its units.	[Delhi BTE, 2017]

- Define the value of absolute permeability.	[Delhi BTE, 2015]

- What is relative permeability?.	[Delhi BTE, 2019]

HINTS TO SHORT ANSWER TYPE QUESTIONS

- Positive and negative charges.

- Coulombs.

- Two poles exists in a magnet, namely North Pole and South Pole.

- Right hand rule.

- Resistance.

- Flux.

- Fleming’s legt hand rule.

- Ampere turns.

- Weber/m2.

- One.

- B = 0r H in medium = 0 H in air.

- mho.

- F = BlI sin  Newtons.

-  BA   NI in air

- SHORT ANSWER TYPE QUESTIONS

  - Define magnetic lines of force and state any one of its property.

  - What is the use of right hand cork screw rule in electromagnetism?

  - What is the direction of magnetic field in a current carrying conductor?

  - Give any two analogy between electric and magnetic circuit.

  - Define m.m.f. and give its unit.

  - What is Fleming’s right hand rule?

  - What is meant by (i) reluctance (ii) permeability?

  - Define electromagnetism and give any two applications of electro magnetism.

  - Define magnetic circuit. How many type of magnetic circuit are generally used in practical life.

  - Make comparison between magnetic and electric circuits.	[PSBTE, 2017, 2015]

  - Define MMF.	[HSBTE, Dec. 2019 ; PSBTE, 2019]

  - What is the analogy between electric and magnetic circuits?	[PSBTE, June 2021]

  - What is electromagnetism? Explain.	[HSBTE, Dec. 2018]

  - Write short notes on Coulomb’s law.	[Delhi BTE, 2018]

  - Explain flux and flux density.	[Delhi BTE, 2018]

  - Explain the magnetic field around a current carrying circular loop.	[Delhi BTE, 2019]

  - Derive an expression for the force acting between two parallel current carrying conductor.

[PSBTE, 2016]

  - Give comparison between electric and magnetic circuits.	[HSBTE, 2017]

  - Define self inductance. What is  it’s S.I. unit?

  - What is the effect on mutual inductance on pair of coils when (i) we increase distance between coils and (ii) we increase number of turns in the coil.

  - Define mutual inductance.

  - Emf induced in a coil is called back emf. Why?

  - State faraday’s laws of electromagnetic induction.

  - How can we find  direction of emf induced in a coil?

  - On what factors emf of conductor depend?

- LONG ANSWER TYPE QUESTIONS

  - State and explain coulombs law of electrostatics.

  - What is magnetic lines of force? Give its properties.

  - Explain the magnetic effect of electric current.

  - Derive an expression for magnetic field due to circular coil.

  - Explain the magnetic field due to a solenoid.

  - Draw and explain the force between two parallel current carrying conductors.

  - Define:

    - M.M.F	(b) Magnetic flux density

(c) Reluctance	(d) Permeability

  - Derive the relation between magnetic flux density permeability and field intensity.

  - With the help of neat sketch explain right hand coak screw rule.

  - With the help of neat diagram explain right hand rule.

  - Give analogy between electric and magnetic circuits for any four points.

  - Write down the applications of a current carrying conductor in a magnetic field.

  - Compare between the similarities and dissimilarities between magnetic and electric circuit.

[Delhi BTE, 2017]

  - State and explain , Faraday’s Laws of electromagnetic induction.

  - What is self inductance? How does it vary with turns, when permeability remains constant?

  - Write notes on the following:

(i)Self and mutually induced emf

(ii)Energy stored in an inductor.

- NUMERICAL QUESTIONS

  - Two spheres charged with equal and opposite charges experience a force of 25 × 105 Newtons,

when they are placed 2 cm apart in a medium of relative permittivity 5. Determine the charge on each sphere.	[Ans. 745 C]

  - Find the force between two charges 10 cm apart in vacuum. The charges are 4 × 10–5 C and 6 × 10– 8 C. If the same charges are separated by the same distance in thinner (r = 2), what is the force between them.	[Ans. 216 N, 1.08 N]

  - If the force between two parallel conductors carrying current in opposite direction is 3.2 N/M, placed at 50 mm apart from one another in air. If the current in one conductor is 1000 amp, find the current in other conductor.	[Ans. Isecond conductor = 800 amp.]

  - Force between two wires carrying currents in opposite direction is 20.4 kg/m when they are placed parallel with the axis 5 cm apart. Calculate the current in one conductor when the current flowing through the second conductor is 5000 amp. Mention whether it is a force of attraction or repulsion.	[Ans. I2 = 10000 amp., force of replusion]

  - Two coils having 100 and 150 turns respectively are wound side by side on a closed iron circuit of section125 cm2 and mean length 200 cm. if the permeability of iron is 2000, calculate (a) self inductance of each coil. (b) mutual inductance between them (c) the emf induced in the second coil if first coil changes from 0 to 5A in 0.02 sec.				[ Ans353.4 mH, 235.6 mH, 58.9 V]

  - The self inductance of a 1000turns coil is 0.3 H. if 70% of the flux is linking with a second coil of 500 turns, calculate (i) the mutual inductance (ii) emf induced in the second coil when the current in the first coil changes at the rate of 250 A per second.		[ Ans. 1.05 H, 262.5 volts]

  - Two coils with a coefficient of coupling of 0.6 between thenm are connected in series so as to magnetise (a) in the same direction (b) in the opposite direction. The corresponding values of total inductances are for (a) 2.96 H and for (b) 1.04 H. Find the self inductances of the two coils and the mutual inductance between them. 				[Ans. 0.48H, 0.64H, L1= 1.6 or 0.4 H, L2= 0.4 or 1.2 H]